# PROJECT REPORT

8-bit Adder/Subtractor using Verilog

Name: SHIVAM

College: R.D. Engineering College

Course: B.Tech (VLSI)

Tool Used: Xilinx Vivado

GitHub: https://github.com/shivamrana0207-dotcom

# 1. Introduction

This project implements an 8-bit adder/subtractor using Verilog HDL. Subtraction is performed using 2’s complement arithmetic and verified using simulation.

# 2. Objective

- Design 8-bit adder/subtractor
- Understand 2’s complement
- Verify using simulation
- Analyze waveform

# 3. Theory

A - B = A + (~B + 1)
Mode = 0 → Addition
Mode = 1 → Subtraction

# 4. Verilog Code (Design)

module adder_subtractor_8bit (

input [7:0] A,

input [7:0] B,

input Mode,

output [7:0] Result,

output CarryOut

);

wire [7:0] B_mod;

wire [8:0] Carry;

assign B_mod = B ^ {8{Mode}};

assign Carry[0] = Mode;

genvar i;

generate

for(i = 0; i < 8; i = i + 1) begin

assign {Carry[i+1], Result[i]} =

A[i] + B_mod[i] + Carry[i];

end

endgenerate

assign CarryOut = Carry[8];

endmodule

# 5. Testbench Code

module tb;

reg [7:0] A, B;

reg Mode;

wire [7:0] Result;

wire CarryOut;

adder_subtractor_8bit uut (

.A(A),

.B(B),

.Mode(Mode),

.Result(Result),

.CarryOut(CarryOut)

);

initial begin

A = 8'd10; B = 8'd5; Mode = 0; #10;

A = 8'd10; B = 8'd5; Mode = 1; #10;

A = 8'd5;  B = 8'd10; Mode = 1; #10;

$stop;

end

endmodule

# 6. Simulation Results

Case 1 (0–10 ns approx):

Case 2 (10–20 ns approx):

Overall waveform:

# 7. Observations

10 + 5 = 15
10 - 5 = 5
5 - 10 = -5 (FB in hex)

# 8. Conclusion

The design successfully performs addition and subtraction operations. Simulation verifies correct implementation.

# 9. Repo link

https://github.com/shivamrana0207-dotcom/8bit-adder-subtractor